## GL17S Lintels – Sheet/Tile N3

| Size (mm) | Sheet Roof Lintels - Roof load Width (m) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 140x42 | 3.6 | 3.0 | 2.6 | 2.3 | 2.1 | 2.0 | 1.8 | 1.7 |
| 190x42 | 4.4 | 3.7 | 3.4 | 3.1 | 2.9 | 2.7 | 2.5 | 2.3 |
| 240x42 | 5.3 | 4.5 | 4.0 | 3.7 | 3.5 | 3.3 | 3.1 | 2.9 |
| 290x42 | 6.0 | 5.1 | 4.6 | 4.3 | 4.0 | 3.8 | 3.6 | 3.4 |
| 140x65 | 3.9 | 3.3 | 3.0 | 2.7 | 2.4 | 2.3 | 2.1 | 2.0 |
| 190x65 | 4.9 | 4.1 | 3.7 | 3.4 | 3.2 | 3.1 | 2.9 | 2.7 |
| 240x65 | 5.8 | 4.9 | 4.4 | 4.1 | 3.8 | 3.6 | 3.5 | 3.3 |
| 290x65 | 6.6 | 5.6 | 5.1 | 4.7 | 4.4 | 4.2 | 4.0 | 3.8 |
| 240x80 | 6.0 | 5.2 | 4.7 | 4.3 | 4.0 | 3.8 | 3.7 | 3.5 |
| Size (mm) | Tile Roof Lintels - Roof load Width (m) |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 140x42 | 2.9 | 2.3 | 2.0 | 1.8 | 1.6 | 1.5 | 1.4 | 1.3 |
| 190x42 | 3.7 | 3.1 | 2.8 | 2.5 | 2.3 | 2.0 | 1.9 | 1.7 |
| 240x42 | 4.4 | 3.7 | 3.3 | 3.1 | 2.9 | 2.6 | 2.4 | 2.2 |
| 290x42 | 5.1 | 4.3 | 3.9 | 3.6 | 3.4 | 3.1 | 2.9 | 2.6 |
| 140x65 | 3.3 | 2.7 | 2.3 | 2.1 | 1.9 | 1.8 | 1.7 | 1.6 |
| 190x65 | 4.1 | 3.5 | 3.1 | 2.9 | 2.6 | 2.5 | 2.3 | 2.1 |
| 240x65 | 4.9 | 4.1 | 3.7 | 3.4 | 3.2 | 3.1 | 2.9 | 2.7 |
| 290x65 | 5.6 | 4.7 | 4.3 | 4.0 | 3.7 | 3.6 | 3.4 | 3.3 |
| 240x80 | 5.1 | 4.3 | 3.9 | 3.6 | 3.4 | 3.2 | 3.1 | 3.0 |

Span values are in metres

Loading Data:

Dead Load of roof and ceiling maximum 90 kg/m2 for tiled roofs, and 40kg/m2 for sheet roofs.
(Covers standard up to terra-cotta roof tiles, plasterboard ceiling below, roof trusses or raftered roof)
Roof Live Load of 0.25kPa.
Wind design for up to N3 wind area, in accordance with AS4055-2006 – Wind Loads for Housing.
ETH LAM GL 17 beams are manufactured straight, without any camber built into the beams.
Lintel design criteria in accordance with methods presented in AS1684.1-1999, and structural timber design in accordance with AS1720.1-2010.
All lintels are designed for single span only.

Notes:

- Minimum bearing lengths for support of lintels: 35mm on trimmer studs. Size and width of trimmer and jamb studs is subject to loadings outside the scope of this table.

- The span value shown is the distance between centrelines of supports.

- Deflection criteria: for permanent load combinations, the lesser of Span/300, or 10mm, and for Roof Live Loads, the lesser of Span/250, or 15mm.

- For lintels the lateral restraint is assumed to be a maximum of 600mm.

- Where there are conflicts in design between loading codes (AS/NZS1170 series), timber code (AS1720.1-2010) and AS1684.1-1999, the loading codes and timber code take preference.

- Maximum roof pitch applicable for these tables, 25 degrees.